DIGITAL IMAGE PROCESSING COURSE - 2021.FALL 
PRACTICE LABS

LAB 03. BINARY IMAGE PROCESSING

Requirements

- Follow the instructions with the help from your instructor.

- Finish all the exercises (given at the end of this document) in class and do the homework at home. You can update your solutions after class and re-submit all your work in the total submission together with the homework.

- Grading
Total score = 50%* In-class submission + 50% * (Total submission) + Homework

- Plagiarism check

If any 2 of the students have the same output images, then all will get zero for the corresponding exercises.

INTRODUCTION

In this Lab, you will learn how to

- Bitwise Operations

- Masking of images

- Alpha Blending with OpenCV

- Image thresholding

INSTRUCTIONS

- Bitwise Operations

OpenCV: Operations on arrays: bitwise_and()

dst = cv2.bitwise_and(src1, src2[, dst[, mask]])

cv2.bitwise_and() is a function that performs bitwise AND processing as the name suggests. The AND of the values for each pixel of the input images src1 and src2 is the pixel value of the output image.

Other Bitwise operation in Python (OR, XOR, NOT, SHIFT)

dst = cv2.bitwise_or(src1, src2[, dst[, mask]])

dst = cv2.bitwise_xor(src1, src2[, dst[, mask]])

dst = cv2.bitwise_not(src[, dst[, mask]])

Source: https://www.journaldev.com/48789/arithmetic-bitwise-and-masking-python-opencv)

Example:

Masking image with a rectangle mask. (image = cv2.imread())

A circle mask

Combine with the AND operation

Bitwise addition refers to the addition of two different images, and decide which is to be displayed using an AND operation on each pixel of the images.

Given a choice with the OR operation

Exclusivity with the XOR operation

Bitwise addition of both the circle and the square

- Masking of images

Masking is used in Image Processing to output the Region of Interest, or simply the part of the image that we are interested in. We tend to use bitwise operations for masking as it allows us to discard the parts of the image that we do not need.

The process of masking images

We have three steps in masking.

- Creating a black canvas with the same dimensions as the image, and naming it as mask.

- Changing the values of the mask by drawing any figure in the image and providing it with a white color.

- Performing the bitwise AND operation on the image with the mask.

Example:

Masking with a rectangle mask.

Masking with a circle mask.

- Alpha Blending with OpenCV

Use cv2.addWeighted() to do alpha blending with OpenCV.

Reference: addWeighted()

dst = cv2.addWeighted(src1, alpha, src2, beta, gamma[, dst[, dtype]])

It is calculated as follows according to parameters. The fifth parameter gamma is the value to be added to all pixel values.

dst = src1 * alpha + src2 * beta + gamma

The two images need to be the same size, so resize them.

import cv2

src1 = cv2.imread('lena.jpg')

src2 = cv2.imread('rocket.jpg')

src2 = cv2.resize(src2, src1.shape[1::-1])

The image is alpha blended according to the values of the second parameter alpha and the fourth parameter beta. Although images are saved as files here, if you want to display them in another window, you can use cv2.imshow() (eg: cv2.imshow('window_name', dst)). The same is true for the following sample code.

dst = cv2.addWeighted(src1, 0.5, src2, 0.5, 0)

cv2.imwrite('opencv_add_weighted.jpg', dst)

Source: https://note.nkmk.me/en/python-opencv-numpy-alpha-blend-mask/

- Image thresholding

The basic Thresholding technique is Binary Thresholding. For every pixel, the same threshold value is applied. If the pixel value is smaller than the threshold, it is set to 0, otherwise, it is set to a maximum value. The different Simple Thresholding Techniques are:

th, dst = cv2.threshold(source, thresholdValue, maxVal, thresholdingTechnique)

## cv2.THRESH_BINARY
If pixel intensity is greater than the set threshold, value set to 255, else set to 0 (black)

## cv2.THRESH_BINARY_INV
Inverted case of cv2.THRESH_BINARY

## cv.THRESH_TRUNC
If pixel intensity value is greater than threshold, it is truncated to the threshold. The pixel values are set to be the same as the threshold. All other values remain the same.

## cv.THRESH_TOZERO
Pixel intensity is set to 0, for all the pixels intensity, less than the threshold value.

## cv.THRESH_TOZERO_INV
Inverted case of cv2.THRESH_TOZERO.

Source: https://learnopencv.com/opencv-threshold-python-cpp/

EXERCISE

Ex2.1. Masking image

Extracting the faces from the input image using a circular mask.

Output is 3 of face images extracted from the input image.

Hint: Using circular masks as in the instructions above.

Download the image via this link

Ex2.2. Blend the following 2 images to create a new image

Image link

Ex2.3. Image thresholding

Given an input image as follows (numbers are pixel intensities)

Download: https://learnopencv.com/wp-content/uploads/2021/06/input_image.jpg

- Convert the image into 02 binary images, that numbers greater than or equals to 180 are in black, and numbers less than 180 are in white.
Hints: Using different thresholds and binarization techniques.

- Extract each number in the input image as in separated images.

HOMEWORK 3

Create a video from your webcam by inserting the TDT logo to images streaming from your webcam.